# Muhammad Amir Mushtaq

# ⚙ 1. SENSORS (10 important sensors for Robotics/IoT)

## 1. Ultrasonic Sensor (HC-SR04)

- Type: Distance sensor (sound waves)

- Range: 2 cm – 400 cm

- Pins: VCC, GND, TRIG, ECHO

- Working: Sends ultrasonic pulse → measures echo time

- Formula:
 Distance = (Time × Speed of sound) / 2

- Power: 5V, ~15mA

- Use: Obstacle avoidance robots

## 2. IR Sensor (Infrared Obstacle Sensor)

- Range: 2 cm – 30 cm

- Pins: VCC, GND, OUT

- Working: IR LED emits light → reflected signal detected

- Output: Digital (0/1)

- Power: 3.3V–5V

- Use: Line follower, obstacle detection

## 3. LDR (Light Dependent Resistor)

- Type: Light sensor

- Range: 0–1000 lux (approx)

- Pins: 2 pins (resistor)

- Working: Resistance decreases when light increases

- Power: Passive (no power needed)

- Use: Light automation systems

## 4. DHT11/DHT22 (Temperature + Humidity)

- Range:

  - Temp: 0–50°C (DHT11), -40–80°C (DHT22)

  - Humidity: 20–90%

- Pins: VCC, GND, DATA

- Working: Digital sensor sends calibrated signal

- Power: 3.3–5V

- Use: Weather stations

## 5. MPU6050 (Gyro + Accelerometer)

- Type: Motion sensor

- Range: ±2g to ±16g, gyro ±250–2000°/s

- Pins: VCC, GND, SDA, SCL

- Working: Measures tilt, acceleration, rotation

- Protocol: I2C

- Use: Drones, balancing robots

## 6. Gas Sensor (MQ-2 / MQ-135)

- Range: Smoke, LPG, CO detection

- Pins: VCC, GND, A0, D0

- Working: Gas changes resistance of sensing layer

- Power: 5V, ~150–200mA (heater inside)

- Use: Safety systems

## 7. PIR Sensor (Motion Detector)

- Range: 3–7 meters

- Pins: VCC, GND, OUT

- Working: Detects infrared heat from humans

- Output: Digital HIGH when motion detected

- Power: 5V

- Use: Security systems

## 8. Soil Moisture Sensor

- Range: 0–100% moisture

- Pins: VCC, GND, A0

- Working: Measures conductivity of soil

- Power: 3.3–5V

- Use: Smart irrigation

## 9. Camera Module (OV2640 / ESP32-CAM)

- Resolution: up to 2MP

- Pins: GPIO interface (ESP32-CAM)

- Working: Captures image/video data

- Power: 5V

- Use: Computer vision systems

## 10. Hall Effect Sensor

- Range: Magnetic field detection

- Pins: VCC, GND, OUT

- Working: Detects magnetic field changes

- Output: Digital/analog

- Power: 3.3–5V

- Use: Speed measurement, motor control

Some sensor hits

| Device | Uses | Internal Parts | Output |
| --- | --- | --- | --- |
| Buzzer | Sound alert | Piezo + oscillator | Digital tone |
| Speaker | Audio/music | Coil + magnet | Analog sound |
| Ultrasonic | Distance | TX + RX + IC | Time signal |
| IR Sensor | Obstacle detection | IR LED + photodiode | Digital/analog |

# ⚙️ 2. ACTUATORS (10 important ones)

## 1. DC Motor

- Voltage: 3V–12V

- Control: H-Bridge (L298N)

- Working: Electrical → rotational motion

- Use: Wheels, robots

## 2. Servo Motor (SG90)

- Angle: 0°–180°

- Pins: VCC, GND, Signal

- Control: PWM signal

- Power: 5V

- Use: Robotic arms

## 3. Stepper Motor

- Step angle: 1.8° typical

- Pins: 4–6 pins

- Control: Pulse sequence

- Use: CNC, precision robots

## 4. Relay Module

- Voltage: 5V control, 220V output

- Working: Electromagnetic switch

- Use: Home automation

## 5. Solenoid Valve

- Type: Linear actuator

- Power: 12V/24V

- Working: Magnetic coil moves piston

- Use: Water/gas control

## 6. Buzzer

- Type: Sound actuator

- Voltage: 3.3–5V

- Use: Alerts, alarms

## 7. LED (Actuator of light)

- Voltage: 2V–3.3V

- Current: 20mA

- Use: Indicators

## 8. Vibration Motor

- Voltage: 3–5V

- Use: Haptic feedback

## 9. Linear Actuator

- Range: 10cm–50cm stroke

- Voltage: 12V/24V

- Use: Robotics movement

## 10. Pneumatic Cylinder

- Working: Air pressure driven

- Use: Industrial robots

# ⚙️ 3. MICROCONTROLLERS (10 important boards)

## 1. Arduino Uno

- CPU: ATmega328P

- Pins: 14 digital, 6 analog

- Voltage: 5V

- Clock: 16 MHz

- Use: Basic robotics

## 2. ESP32

- CPU: Dual-core 240 MHz

- Pins: 34 GPIO

- WiFi/Bluetooth: Yes

- Voltage: 3.3V

- Use: IoT + AI edge devices

## 3. ESP8266

- WiFi: Yes

- Pins: Limited GPIO

- Use: IoT projects

## 4. Raspberry Pi 3/4/5

- Type: Mini computer

- RAM: 2GB–8GB

- OS: Linux or Raspberry Pi OS

- Use: AI + Computer vision

## 5. STM32

- CPU: ARM Cortex-M

- Speed: High performance

- Use: Industrial systems

## 6. PIC Microcontroller

- Use: Embedded systems

- Low power applications

## 7. Arduino Mega

- Pins: 54 digital, 16 analog

- Use: Complex robotics

## 8. Jetson Nano (NVIDIA)

- AI GPU enabled

- Use: Deep learning robotics

## 9. BeagleBone Black

- Linux embedded board

- Use: Industrial automation

## 10. Teensy Board

- High-speed microcontroller

- Use: Audio + robotics